XMC1302 based eDisconnect Board Firmware

XMC1302 Firmware

Contact

- Nyan Paing Soe (IFAP DC PSS SI PS INNO)

- Seet Wei Ren Ivan (IFAP DC PSS SI PS TM)

About this document

Scope and purpose

This document describes the firmware implementation on XMC1302 to control eDisconnect Board.

Intended Audience

Anyone working with 2ED4820 gate driver and XMC1302.

Table of contents

Contact	1

About this document	1

Table of contents	2

1	Overview	4

1.1	Block Diagram of eDisconnect switch	4

1.2	Development tools	4

1.3	External library	4

2	Requirement and Specifications	5

3	XMC1302 Interaface	6

4	Dave Apps parameter settings	7

4.1	List of Dave App	7

4.2	APP Configuration and parameter settings	8

4.2.1	Digital IO	8

4.2.2	PWM CCU 4	8

4.2.3	PWM_0	10

4.2.4	INTERRUPT	11

4.2.5	ADC_MEASUREMENT	11

4.2.6	SYSTEM TIMER	13

4.2.7	SPI COMMUNICATION	13

5	Firmware Module	17

5.1	SYS TIMERs and IRQ Handlers	17

5.2	Operation State Diagram	18

5.3	Protection	20

5.4	SPI Communication and 2ED4820 Gate Driver register	21

5.5	Firmware Structure in Dave IDE	22

6	ADC sensing and processing	23

6.1	Voltage Sensing	23

6.2	Current sensing	24

6.3	Temperture sensing	25

7	System protection	27

7.1	Precharge switch	27

7.2	Freewheeling switch	29

8	GUI	30

9	Hardware Schematic	31

References	34

Revision history	35

Figure 1 Block diagram of eDisconnect switch	4

Figure 2 PWM CCU4 PWMA general setting	8

Figure 3 Hardware connection of PWMA event to PWMB	9

Figure 4 PWMB General setting	9

Figure 5 PWMB External Event Setting	9

Figure 6 PWM_0 General setting	10

Figure 7 PWM_0 Event setting	10

Figure 8 PWM_0 Pin setting	10

Figure 9 PWM_0 Hardware connection to INTERRUPT_1 irq	11

Figure 10 INTERRUPT_1 setting	11

Figure 11 ADC_MEASUREMENT_0 General setting	12

Figure 12 ADC_MEASUREMENT_0  Measurement setting	12

Figure 13 ADC_MEASUREMENT_0 Interrupt setting	12

Figure 14 SYSTIMER General setting	13

Figure 15 SPI DRIVER General setting	13

Figure 16 SPI DRIVER Advanced setting	14

Figure 17 SPI DRIVER Interrupt setting	15

Figure 18 SPI DRIVER Pin Setting	16

Figure 19 Timing diagram for Handlers	17

Figure 20 Block diagram for System timer and handler	18

Figure 21 Operation state diagram	19

Figure 22 Flowchart for system level protection	20

Figure 23 2ED4820 Register overview	21

Figure 24 Firmware structure in Dave IDE	22

Figure 25 Voltage sensing for battery and load voltage	23

Figure 26 Low side Shunt current sensor through 2ED4820	24

Figure 27 Thermistor circuit	25

Figure 28 Plot of Temperature vs ADC voltage and ADC values	25

Figure 29 RT characteristics of B57861S	26

Figure 30 Temperature and ADC range	26

Figure 31 Precharge and Freewheel setting in GUI	27

Figure 32 Flowchart for Precharge algorithm	28

Figure 33 Timing diagram for Freewheel operation	29

Figure 34 Captured freewheel signal (CH1: Freewheel signal, CH2: Load voltage, CH3: delay signal)	29

Figure 35 GUI for eDisconnect switch in MicroInspector	30

# Overview

The eDisconnect board Firmware is developed to control Infineon microcontroller XMC 1302 and smart high side MOSFET gate driver 2ED4820 to operate high power switch at battery terminal. Flexible precharge method and protection are also implemented in this firmware.

## Block Diagram of eDisconnect switch

Figure 1 Block diagram of eDisconnect switch

## Development tools

The firmware is developed with Dave IDE from Infineon developer center. The GUI is developed and operate with MicroInspector. XMC link debugger probe is required.

## External library

The firmware uses 2ED4820 Library which is mentioned in session 5.5.

# Requirement and Specifications

Table 1 Specification of eDisconnetion switch

| Parameter |  | Min. | Typ. | Max. | Peak | Unit | Note | Req. Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cell Voltage |  | 2.6 | 3.6 | 4.2 | 4.25 | V | LG Li-ion INR21700M50L 4800mAh |  |
| Voltage | 14S7P | 35 | 48 | 58.8 | - | V | Programmable to choose the battery Configuration | HW,SW |
| Current (cont.) | 14S7P | - | 54 | 100 | - | A | @ TJ,max = 100⁰C | HW,SW |
| Output Voltage |  | 35 | 48 | 58.8 | - | V |  | HW |
| Precharge Time |  | - | 10 | 1000 | - | ms |  | SW |
| Precharge Current |  | - | 5 | - | 33.6 | A |  | SW, HW |
| Over Voltage (OV) |  | - | - | 60 | - | V |  | SW |
| Under Voltage (UV) |  | - | - | 30 | - | V |  | SW |
| Over Current (OC) |  | - | - | 130 | - | A | 130% of 3C Discharge | SW |
| Transient Current (TC) |  | - | - | 150 |  | A | 150% @ t=200us | SW |
| Short Circuit Current |  | 150 | - | 3000 | TBD | A |  | SW |
| Temperature |  | 15 | - | 80 | 100 | °C |  | SW,HW |
| Response time |  | 1 | - | 50 | TBD | us |  | SW |
| VAUX |  | 3.3 | 3.3 | 5 | - | V | VDD for CS, MCU | HW |
| Gate drive current |  | - | - | 1A | TBD | A |  | HW |
| SPI/RS485 |  |  |  |  |  |  | CS gain, OC threshold, GD control, diagnosis | SW |
| JTAG |  | 1.27mm 10-pin debug connector |  |  |  |  | Microcontroller Interface | SW |
| LED |  | RED, GREEN/YELLOW, AMBER |  |  |  |  | 3-pin RGB for indications | HW,SW |
| Heat Sink (YES/NO) |  | Rth = 5K/W (approx) |  |  |  |  | ULP/LPD80/N80/Al | HW |
| Size |  | Max: 86mm x 41mm x 33mm |  |  |  |  |  |  |

# XMC1302 Interaface

Table 2 XMC1302 Pin out

| Pin | Port | Signal | Description |
| --- | --- | --- | --- |
| 1 | P2_6 | OCD2_GPIO | For TLE4972 current programmer (Not use in V2.2) |
| 2 | P2_7 | OCD1_GPIO | For TLE4972 current programmer (Not use in V2.2) |
| 3 | P2_8 | NTC_ADC | Thermistor reading |
| 4 | P2_9 | I_SNS_ADC | Current sense from 2ED4820 |
| 5 | P2_10 | VBAT_SNS_ADC | Battery voltage |
| 6 | P2_11 | VL_SNS_ADC | Load voltage |
| 7 | VSS | SGND | Ground |
| 8 | VDD | VDDP/VDD | 3.3V |
| 9 | P1_3 | FW_Ctrl | Freewheeling control signal |
| 10 | P1_2 | PWM_A | Freewheeling timer delay (output for debug only) |
| 11 | P1_1 | PC_Ctrl | Precharge control signal |
| 12 | P1_0 | PWM_B | Not used |
| 13 | P0_0 | SAF_GPIO | Safestate Enable pin to 2ED4820 |
| 14 | P0_4 | EN_GPIO | Enable pin to 2ED4820 |
| 15 | P0_5 | PX. RESET | Reset pin |
| 16 | P0_6 | MOSI_GPIO | SPI communication |
| 17 | P0_7 | MISO_GPIO | SPI communication |
| 18 | P0_8 | SCLK_GPIO | SPI communication |
| 19 | P0_9 | CSN_GPIO | SPI communication |
| 20 | P0_10 | PX_LED_R | Red status LED |
| 21 | P0_12 | PX.LED.G | Green status LED |
| 22 | P0_13 |  |  |
| 23 | P0_14 | SWIO_uC | Debugger date |
| 24 | P0_15 | SWCLK_uC | Debugger clock |
| 27 | P2.1 | VREF_ADC | For TLE4972 current programmer (Not use in V2.2) |

# Dave Apps parameter settings

## List of Dave App

The following table lists the number of Dave app used in firmware. Whenever the changes are made in these app, it is required to generate the code first before compiling main firmware.

Table 3 List of Dave app used in this firmware v 2.2

| Instance Label | Dave App | Output Pin | Description |
| --- | --- | --- | --- |
| PX_LED_G | DIGITAL_IO | P0.12 | Green LED Indicator |
| PX_LED_R | DIGITAL_IO | P0.10 | Red LED Indicator |
| IO_DRIVER_ENABLE | DIGITAL_IO | P1.0 | 2ED4820 enable |
| IO_DRIVER_SAFESTATEEN | DIGITAL_IO | P0.4 | 2ED4820 Safestate enable |
| PWMA | PWM_CCU4 | P1.2 | Short timer before freewheel signal is ON. (~50us) |
| PWMB | PWM_CCU4 | P1.3 | Freesheel output pulse after switch is off |
| PWM_0 | PWM | P2.0 | Precharge Signal |
| TIMER_0 | TIMER |  | 1ms timer (no longer used in v2.2) ** |
| INTERRUPT_0 | INTERRUPT |  | timer 0 interrupt (no longer used in v2.2) ** |
| INTERRUPT_1 | INTERRUPT |  | PWM_0 periodic event |
| PIN_INT_DRIVER | PIN INTERRUPT | P2.0 | Fault Interrupt from 2ED4820 |
| ADC_MEASUREMENT_0_CH0 | ADC_MEASUREMENT | P2.9 | Current sensor |
| ADC_MEASUREMENT_0_CH1 | ADC_MEASUREMENT | P2.8 | Temperature sensor |
| ADC_MEASUREMENT_0_CH2 | ADC_MEASUREMENT | P2.10 | Battery voltage |
| ADC_MEASUREMENT_0_CH3 | ADC_MEASUREMENT | P2.11 | Load (Pack) voltage |
| ADC_MEASUREMENT_0_CH4 | ADC_MEASUREMENT | P2.1 | Vref (current sensor) |
| SYSTIMER_0 | SYSTIMER |  | 3 system timers (5ms, 10ms and 50ms) |
| SPI_DRIVER (MISO) | SPI_MASTER | P0.7 | SPI MISO |
| SPI_DRIVER (MOSI) | SPI_MASTER | P0.6 | SPI MOSI |
| SPI_DRIVER (SCLKOUT) | SPI_MASTER | P0.8 | SPI CLK |
| SPI_DRIVER (Slave Select) | SPI_MASTER | P0.9 | SPI Slave select |

** It is possible to remove in upcoming version

## APP Configuration and parameter settings

### Digital IO

Table 4 Digital IO parameters

| Digital IO | Pin Direction | Mode | Init Level |
| --- | --- | --- | --- |
| PX_LED_G | Input/Output | Push Pull | Low |
| PX_LED_R | Input/Output | Push Pull | Low |
| IO_DRIVER_ENABLE | Input/Output | Push Pull | Low |
| IO_DRIVER_SAFESTATEEN | Input/Output | Push Pull | High |

### PWM CCU 4

PWM CCU 4 module has two PWM signals (PWM A and PWM B) for freewheeling implementation. Both PWM A and PWM B are configured with single shot mode operation as it only requires one pulse after MOSFET switch is off. PWM A serves as delay timer 50us during MOSFET turn off. The falling edge of PWM A creates external event to trigger PWM B for freewheeling action.

Figure 2 PWM CCU4 PWMA general setting

Figure 3 Hardware connection of PWMA event to PWMB

Figure 4 PWMB General setting

Figure 5 PWMB External Event Setting

### PWM_0

PWM_0 signal is produced for Pre charge operation. The signal is fed to linear MOSEFT to turn on with signle pulse mode or PWM mode. The pre charge mode can be selected for different application requirement. In event settings, PWM_0 has periodic match interrupt connected to INTERRUPT_1.

Figure 6 PWM_0 General setting

Figure 7 PWM_0 Event setting

Figure 8 PWM_0 Pin setting

Figure 9 PWM_0 Hardware connection to INTERRUPT_1 irq

### INTERRUPT

This INTERRUPT_1 app called when PWM_0 periodic match event occurs. The function UserIRQHandler is called for PWM_0 event clearance and pulse counting for Pre charge function.

Figure 10 INTERRUPT_1 setting

### ADC_MEASUREMENT

ADC measurement is set up with continuous conversion. The desired sampling is set as 1000 ns. There are 5 ADC channels for measurement of the following.

Table 5 ADC measurement signals and ports

| ADC Signals | Port | Value |
| --- | --- | --- |
| VBAT_SNS | P2.10 | Battery Voltage |
| VL_SNS | P2.11 | Load (Pack) voltage |
| I_SNS_ADC | P2.9 | Current sensor (2ED4820) |
| VREF_ADC | P2.1 | REF voltage for TLE4972 current sensor |
| NTC_ADC | P2.8 | Temperature sensor |

At the end of measurement, ADC_measurement handler interrupt is called to notify the new meansurment are available.

Figure 11 ADC_MEASUREMENT_0 General setting

Figure 12 ADC_MEASUREMENT_0  Measurement setting

Figure 13 ADC_MEASUREMENT_0 Interrupt setting

### SYSTEM TIMER

Figure 14 SYSTIMER General setting

### SPI COMMUNICATION

Figure 15 SPI DRIVER General setting

Figure 16 SPI DRIVER Advanced setting

Figure 17 SPI DRIVER Interrupt setting

Figure 18 SPI DRIVER Pin Setting

# Firmware Module

## SYS TIMERs and IRQ Handlers

The system timer is set up with 1ms tick. There are 3 system timers which are called periodically to impletement the entire firmware module. Each timer is assigned to call different handlers.

Table 6 System Timer

| SYSTIMER ID | Timer event mode | Timer value | SYSTIMER Call Handler |
| --- | --- | --- | --- |
| TimerId_1 | Periodic | 10ms | Operation |
| TimerId_2 | Periodic | 50ms | GUI |
| TimerId_3 | Periodic | 5ms | Fault |

Figure 19 Timing diagram for Handlers

The fault handler is called at every 5ms and check the status of overall system quickly and update LED status for monitoring.

The operation handler is called at every 10 ms. It is the core of the eDisconnect switch operation. Due to continuous conversion of ADC, the new raw values are available at every handler call. The measurement values are processed to get actual reading (voltage, current, etc) in this routine. Besides, the operation state diagram (5.2) is also implemented.

The GUI handler is called at every 50ms. The handler reads the current button states and values from GUI and updated. It also provides the function to pass those button state to operation handlers for making decision in the state diagram. It also extracts data from the other handlers and prepare the values to send so that GUI can display as required.

Figure 20 Block diagram for System timer and handler

## Operation State Diagram

There are 4 operation states depending on the enable switch, safestate, user input switch on/off and fault.

Init State, OpState = 0:  Whenever the enable signal goes low, the system will be in init state. In the version 2.2, the state is bypassed (ie, enable = 1).

OFF state, OpState = 1: The eDisconnect switch is OFF in this state. The freewheel protection is included. In the event of switch off, freewheel enable and completion is checked. When the freewheel is enable and complete flag is false, the freewheel protection is executed first before sending switchoff signal to MOSFET gate driver via register. The freewheel completion flag is set to 1 if the switch is successfully turned off. When freewheel protection is disabled, it will be switch off immediately. Unless there is fault or user input to turn on, the switch will remain OFF state.

On state, Opstate = 2: The eDisconnect switch can be turned on by user switch input. During switch on event, the precharge enable and completion is checked. Precharge configuration is called and calculate the maximum pulses needed for precharge operation according to the user settings. When precharge mode is enabled and completion flag is false, the precharge operation is carried out before the turning on the MOSFET.   Unless there is fault or user input to turn off, the switch will remain ON state.

Fault state, Opstate = 3: The eDisconnect switch will go to OFF state immediately after the fault trigger flag is 1 or problem detected in gate driver. During the fault state, the switch is forced to turned off. The user is not allowed to switch on until fault is cleared and returned to OFF state.

Figure 21 Operation state diagram

## Protection

The firmware implements several protections in the system level such as overvoltage, undervoltage and over/under temperature. The fault check function is called at every 5 ms.

To determine the fault condition, the maximum and minimum threshold values can be referred to the specification table in session 2.

Figure 22 Flowchart for system level protection

## SPI Communication and 2ED4820 Gate Driver register

The SPI slave is provided by 2ED4820 gate driver. Therefore, in this firmware, SPI master module must be used with 16-bit protocol. The SPI read and write functions are provided by DRIVER_2ED4820EM library. The SPI master is set up with Dave app. The parameters and configurations are described in session 4.2.7. The operation and diagnosis of the gate driver is done by accessing and updating the register content via SPI communication.

The Register STDIAG, CHDIAG and DIAG is read only diagnosis register. When interrupt pin is set to 1, microcontroller reads the status of these registers and updates in GUI. The microcontroller then cleans the failure by setting register FAILURE CLEAN and check if the bit 7 of STDIAG is cleared. Otherwise, the eDisconnect switch is set to fault state.

Figure 23 2ED4820 Register overview

The eDisconnect switch can be turned and off respectively by setting and clearing of MOSONCH_A and MOSONCH_B of MOS_CHS_CTRL register. All the register bits can be set by using the function call provided by gate driver library.

## Firmware Structure in Dave IDE

Figure 24 Firmware structure in Dave IDE

# ADC sensing and processing

## Voltage Sensing

The battery voltage (VBAT_SNS_ADC) and load voltage (VL_SNS_ADC) are sensed with voltage divider network.

Figure 25 Voltage sensing for battery and load voltage

The resistor divider value is 2.7k/(2.7k+51k) = 0.05.

To measure 48V, the voltage at VBAT_SNS_ADC is (48x0.05) = 2.4V which corresponds to ADC value 2978 for 12- bit ADC. Therefore, to calculate gain to convert voltage reading in microcontroller,

Voltage gain = (3.3/4096)/0.05 = 0.016

## Current sensing

The lower-side current sense configuration is used in this eDisconnect switch. The shunt value is 0.05mOhm. The shunt resistor voltage is filtered and amplified by the internal amplifier provided by 2ED4820. The CSO value is connected to ADC of microcontroller through RC low pass filter with value 1kOhm and 10nF.

Figure 26 Low side Shunt current sensor through 2ED4820

The current value is computed by the following steps.

- Vcso=IADC*(3.3/4096)                                                      // IADC: 0 to 4096(12 bits)

- Shunt Voltage=(Vcso-(VDD/2))/CSAG                // CSAG = Gdiff = 40V/V

- Shunt Current=Shunt Voltage/Rshunt                    // Rshunt= 50uOhms

- yn=1Ni=0N-1x[n-i]                                                            // Moving Average Filter (Shunt Current) N=50

- yn=Ki*y[n]                                                                        //Ki = gain adjustment

Vcso is computed with ADC gain and the shunt voltage is calculated by current sense gain CSAG = 40 configured in gate driver register. The shunt current is computed with Rshunt = 50uOhm. The computed value is furthered smoothed by using moving average filter with N = 50. The arbitrary value Ki is the gain adjustment to be made experimentally in this firmware version.

## Temperture sensing

B57861S NTC thermistor is used to sense ambient temperature around eDisconnect switch. The 10k NTC is connected to 10k pull up resistor which form voltage divider circuit at NTC_ADC. The lookup table is developed to translate the measured ADC values into temperature reading to show in GUI.

Figure 27 Thermistor circuit

The charactesistics of NTC is plotted with normalized RT/R25 vs temperature.

The ADC values are then computed according to analog circuit network for the temperature value range from 0 degree to 155 degree Celsius. A good linearity is observed from 0 degree to 60 degree Celsius.

Figure 28 Plot of Temperature vs ADC voltage and ADC values

Figure 29 RT characteristics of B57861S

Figure 30 Temperature and ADC range

# System protection

## Precharge switch

The precharge switch is enabled by default and it can be disabled by user via GUI. There are two precharge mode implemented in this firmware. The signle pulse mode or PWM mode can be selected. The selection table and GUI is shown.

Table 7 Precharge mode selection

| Precharge | Mode | Frequency (Hz) | Duration (ms) |
| --- | --- | --- | --- |
| Enable | Single pulse | NA | 100 |
|  |  | NA | 250 |
|  |  | NA | 500 |
|  | PWM | 10 | 100 |
|  |  | 10 | 500 |
|  |  | 10 | 1000 |
|  |  | 50 | 100 |
|  |  | 50 | 500 |
|  |  | 50 | 1000 |
|  |  | 100 | 100 |
|  |  | 100 | 500 |
|  |  | 100 | 1000 |
| Disable | NA | NA | NA |

Figure 31 Precharge and Freewheel setting in GUI

The precharge algorithm is called in the operation ON state (OpState = 2). When Precharge enable button is turned ON, the precharge completion flag is checked. If the flag is not set, the precharge algorithm is called. The condition for precharge completion is determined by the maximum pulse count generated by PWM_0. This is applicable for both single pulse and PWM mode. Depending on the selection, the maximum pulse count is determined by precharge configuration function. The precharge failure check is not implemented in this firmware version 2.2 and before.

Figure 32 Flowchart for Precharge algorithm

## Freewheeling switch

The freewheeling is enabled by default. It can be disabled by user via the freewheel enable button in GUI. The freewheel routine is called during switch off operation (OpState = 1).

Figure 33 Timing diagram for Freewheel operation

When the MOSFET is turned off, after brief duration of delay (50us), the freewheel signal pulse is generated for 500us. The two PWM CCU4 is used for this operation. Firstly, PWMA is generated with predefined frequency and duty cycle to achieve delay of 50us. The PWMA falling event is captured and activate the start of PWMB for another 500 us.

Figure 34 Captured freewheel signal (CH1: Freewheel signal, CH2: Load voltage, CH3: delay signal)

It should be noted that the PWMA pulse time does not represent actual delay but less than 50us. To achieve delay approximately 50us, the 50kHz frequeny with 50% duty is used.

# GUI

The user interface GUI is built with microInspector. To use GUI, XMC link isolated debugger is required. The system state shows the battery voltage, load voltage, current, and temperature value. The user can use SW ON/OFF button to turn on and off eDisconnect switch. LED RED and GREEN are displayed to align the LED status in the PCB board. Precharge and freewheel setting is also included in the GUI.

Figure 35 GUI for eDisconnect switch in MicroInspector

# Hardware Schematic

# References

[1]

Revision history

| Document version | Date of release | Description of changes |
| --- | --- | --- |
| V1.0 | 2023-10-18 | First draft version |